Issues, Causes and Fixes

| Issue | Possible Causes | Fixes |
| --- | --- | --- |
| Motor & propeller won’t turn even when motor & propeller are removed from car | Battery connections are bad – metal surfaces aren’t touching. Test them with a digital multimeter. See the Appendix, Part 4. | Review how to make connections – see “Connecting to an AA battery” under Troubleshooting, Lesson 3 |
|  | Switch or connection to switch is not working. Test them with a digital multimeter. See the Appendix, Part 7 | Review requirements for a switch & types of switches, Troubleshooting, Lesson 2. |
|  | Battery is dead. Test it with a digital multimeter. See the Appendix, Part 3. | Test battery by connecting a motor that you know is good directly against the battery terminals. If the good motor still doesn’t come on, the battery must be dead. Replace it. |
| Motor & propeller turns when removed from car, but not when attached to car | Propeller is hitting other parts of the car | Move motor & propeller so it isn’t hitting |
|  | Propeller is hitting the ground | Raise motor and propeller so they don’t hit |
| Propeller and motor come loose when attached to car | Motor is not attached tightly enough | Use more tape or make a pocket in the car body for holding motor |
| Propeller rotates but does not drive car | Car is too heavy | Remove material or use lighter materials |
|  | Too much friction between wheels and body | Allow wheels to spin freely, as in Lessons 4 & 5 |
|  | Car body obstructs air flow | Remove material from behind propeller |
| Car goes in circles | Axles are not parallel | Reposition axles to make them parallel |
|  | More friction on one side | Check to see that wheels can turn equally freely on both sides; if not , reduce friction on slow side |
| Car goes backwards | Motor is turning in the wrong direction | Reverse the wires from the motor, or to the battery (see Lesson 1, step 3) |
| Car tips over | Weight is unbalanced | Add weights, such as steel washers; move battery; or redesign body so weight is better distributed |